HiClimaX: A High-resolution Climate Downscaling Multi-Approach System

Quang-Van Doan1,

1Center for Computational Sciences, University of Tsukuba, Tsukuba, 305-0006, Japan

2Laboratory example, city, postal code, country

Correspondence to: Quang-Van Doan (doan.van.gb@u.tsukuba.ac.jp)

Abstract. Urban areas are at the forefront of climate change impacts. Fine-scale urban climate change predictions remain a challenge due to the complexity of urban environments and uncertainties in climate modeling. This study introduces HiClimaX (A High-resolution Climate Downscaling Multi-Approach System)—the first-ever comprehensive toolbox designed to generate high-resolution climate change projections tailored cities. By integrating multiple state-of-the-art approaches, including i) direct dynamical downscaling; ii) pseudo-global warming; iii) statistical dynamical downscaling; iv) land-surface-physics-based downscaling; v) statistical downscaling vi) AI-based downscaling, HiClimaX offers a streamlined, accessible, extendable and transparent framework. The core innovation of HiClimaX is to bring together diverse approaches into ensemble framework. This implimentation ensures robust and reliable outputs, effectively “storylining” climate change scenarios for urban settings, providing stakeholders with actionable insights tailored to urban adaptation and mitigation strategies. This pioneering framework marks a significant step forward in bridging the gap between global climate models and urban-scale applications, ensuring cities are equipped to thrive in a changing climate.

# 1 Introduction

Urban areas are increasingly recognized as critical hotspots of climate change impacts. More than half of the world’s population already lives in cities, and this proportion is expected to rise to more than two-thirds by mid-century (UN-Habitat, 2020). Cities bring together dense populations, complex infrastructure, and concentrated economic activity. This concentration makes them particularly vulnerable to climate hazards including heatwaves, extreme rainfall, flooding, and tropical cyclones (IPCC AR6, 2022). The interaction between global warming and local urban processes generates additional complexity. Urban heat islands amplify background warming during extreme events like heatwaves (Xue et al., 2024), land–atmosphere feedbacks reshape precipitation patterns (Shepherd and Burian, 2003; Doan et al., 2021), and cascading effects across energy, health, and transport systems intensify risks (Oke et al., 2017).

There is critical gap between the urgency of cities to adapt and mitigate climate risks and the accessibility to reliable and actionable climate information at the scale of cities. Global climate models (GCMs) are designed for planetary to continental scales, with horizontal resolutions of 25–100 km (Iles et al., 2020; Mishra et al., 2023). At such coarse scales, urban forms and heterogeneity cannot be represented explicitly. To address these gaps, a wide range of downscaling methods have been developed (Wilby and Dawson, 2013; Rummukainen, 2016). Dynamical, statistical, and hybrid approaches each provide detailed information and insights into regional climate change, yet having their own advantages and limitations (Wilby and Dawson, 2013). Important to note, they are typically applied in isolation, limiting opportunities for systematic comparison, ensemble integration, or transparent treatment of methodological uncertainty.

Progress in urban climate downscaling has been constrained by several structural gaps. A first limitation is the fragmentation of methods. Many studies depend on one or two approaches, such as dynamical downscaling with a regional climate model (Hamdi et al., 2014; Devadarshini et al., 2025) or statistical regression from reanalysis predictors (Gutiérrez et al., 2013; Tabari et al., 2021). These isolated applications provide useful case-specific results but do not support cross-comparison, assessment of robustness, or systematic biases/uncertainties (Zhang et al., 2020; Lafferty and Sriver, 2023) . Without a common framework, the resulting knowledge fragmented and piecemeal, limiting its utility for broader scientific synthesis and for informing policy and adaptation planning.

A second limitation is the incomplete treatment of uncertainty. Following Shepherd (2018), downscaling uncertainty can be separated into aleatoric—the chaotic variability of the climate system, partly addressed by ensembles—and epistemic, arising from methodological assumptions (Shepherd et al., 2018). Different families of methods introduce distinct epistemic uncertainties: dynamical models depend on parameterizations and resolution, statistical approaches on predictor choice and stationarity, and machine learning on architecture, training data, and generalizability. Each also inherits uncertainties from emissions scenarios, GCM forcings, and reanalysis products. Most workflows rely on one or two approaches applied in isolation, which leaves key uncertainties unacknowledged and inconsistently addressed. This practice produces fragmented results that hinder systematic comparison and reduce confidence in the outcomes.

A third barrier is the poor representation of urban complexity. Cities are not homogeneous surfaces but mosaics of land-cover types, including impervious surfaces, vegetation, and water bodies (Grimmond et al., 2010; Oke et al., 2017). They also exhibit diverse building morphologies that shape energy exchanges and airflow ((Kusaka et al., 2001; Martilli et al., 2002; Doan and Kusaka, 2019), along with anthropogenic heat emissions from transport, industry, and energy use (Sailor et al., 2015). These features exert strong influences on local climate, particularly during extremes such as heatwaves (Stone et al., 2010) and convective storms (Shepherd and Burian, 2003; Doan et al., 2021). Yet many downscaling systems omit or oversimplify them, either due to limited datasets or additional modelling requirements (Masson et al., 2020). As a result, simulated climates often fail to reproduce processes critical for informing urban adaptation strategies.

A fourth barrier is computational and data constraints. High-resolution RCMs with urban canopy models are among the most physically realistic tools available, but they demand massive computational resources. This cost restricts their application to a handful of case studies rather than systematic ensemble runs. By contrast, statistical and AI-based methods are efficient and scalable, but they often depend on local datasets that are inconsistent or difficult to access, making broad application challenging. Finally, limited accessibility and transparency continue to hinder progress. Many workflows are developed for specific projects, with undocumented preprocessing steps, ad hoc postprocessing, and restricted data access. As a result, reproducibility is low and transferability is limited, reducing confidence and uptake by researchers, policy-makers, and practitioners.

These barriers point to the urgent need for a unified, modular, and open framework for urban climate downscaling. Such a system must be capable of integrating multiple approaches, explicitly quantifying uncertainties, incorporating detailed urban surface data, and providing reproducible workflows accessible to diverse users. Only by meeting these requirements can downscaling deliver credible, transparent, and actionable information to the urban climate research community and beyond.

In this study, we present HiClimaX, the High-resolution Climate Downscaling Multi-Approach System. HiClimaX has been designed as a modular and extensible framework that integrates six complementary downscaling methods. These are (1) direct dynamical downscaling, (2) pseudo-global warming experiments, (3) statistical–dynamical downscaling, (4) land-surface-physics-based downscaling, (5) statistical downscaling, and (6) AI-based downscaling. Each module addresses a distinct dimension of the downscaling challenge. Dynamical models provide physically consistent simulations of mesoscale processes. Pseudo-global warming experiments allow isolation of climate-change signals during high-impact events. Statistical–dynamical methods combine circulation typing with empirical correction for efficiency. Land-surface-physics-based approaches emphasize the role of urban morphology and anthropogenic heat fluxes. Statistical approaches allow rapid production of long time series. AI-based methods exploit nonlinear pattern recognition to emulate high-resolution fields with reduced computational costs. By combining these diverse modules, HiClimaX provides a systematic framework for explicit intercomparison and ensemble generation.

The strength of HiClimaX lies in this integration. Outputs from different modules are standardized and can be combined into ensembles that make methodological differences explicit. Ensemble products include mean fields, spread statistics, probability distributions, and storyline-based representations that highlight plausible climate futures for cities. The framework is designed for transparency and reproducibility, with all preprocessing and postprocessing steps scripted, and relies on widely available datasets for forcing and evaluation. This openness facilitates replication, encourages collaboration, and lowers barriers for adoption across different regions.

HiClimaX thus represents a step change for urban climate downscaling. It moves the field from isolated, method-specific studies toward a coherent, transparent, and ensemble-based practice. By bridging the gap between global climate models and city-scale applications, it provides a tool for systematic research, intercomparison exercises, and the delivery of actionable climate information. The framework is not only a research contribution but also a foundation for climate services, urban digital twins, and policy-relevant applications that demand reliable projections of climate change at the scale where most people live.

# 2 Conceptual Framework

HiClimaX has been conceived as a modular and open-source system with the ambition to deliver high-resolution climate information that can be applied in urban contexts. By bringing together several methodological families into one common framework, HiClimaX provides a possibility to compare directly the performance of different approaches, to combine their outputs into ensembles, and to evaluate uncertainties in a more systematic way.

The design philosophy of HiClimaX is that each downscaling method should exist as an independent module/repository, but all share standardized inputs and outputs. In this way, modules can operate separately, yet their results remain directly comparable. Such standardization also lowers the barrier for reproducibility, because the same preprocessing and postprocessing routines can be applied regardless of which module is used. This makes the framework less dependent on project-specific solutions and allows it to function as a general platform across different cities, climates, and datasets.

The overall architecture of HiClimaX is built on three pillars. The first pillar is the data interface, which is responsible for downloading and preparing a wide range of datasets, including reanalysis products, GCM outputs, city-scale land-surface information, and observations. These data are harmonized to common formats, grid resolutions, and variable conventions, which otherwise are a frequent obstacle in urban modelling. The second pillar is the suite of downscaling modules, which presently consists of six families: direct dynamical downscaling, pseudo-global warming experiments, statistical–dynamical methods, land-surface-physics-based schemes, conventional statistical approaches, and AI-driven models. Together, they provide a balance between physical realism, computational efficiency, and flexibility in data demands. The third pillar is the postprocessing and integration layer, where bias corrections, ensemble calculations, and visualization routines are applied. This layer ensures that results are not only technically consistent but also usable for impact studies and stakeholder communication.

An important characteristic of HiClimaX is its extensibility. New modules or new datasets can be incorporated with relatively small effort, as long as they follow the standardized interfaces. For example, if a new AI architecture is developed, it can be added as another module without disturbing the existing system. Similarly, new urban morphological datasets can be introduced in the preprocessing routines with limited modification. This property makes HiClimaX future-proof and relevant for both scientific exploration and practical climate services.

### 2.1 Downscaling modules / repositories

The central innovation of HiClimaX lies in its integration of six downscaling families, each of which has been widely used in the climate community but rarely combined in a systematic way.

### (a) Direct dynamical downscaling

Direct dynamical downscaling employs regional climate models (RCMs), such as the Weather Research and Forecasting model or RegCM, to generate physically consistent fields at kilometre-scale resolution (Skamarock et al., 2008; Giorgi, 2019). By using boundary conditions from global climate models (GCMs) or reanalyses, these simulations dynamically resolve atmospheric processes at 1–5 km grid spacing. At this scale, RCMs can represent mesoscale features critical for cities, including sea breezes, convective storm development, and land–atmosphere feedbacks (Prein et al., 2017).

A distinctive advantage of this approach is its physical consistency. All variables—temperature, wind, precipitation, surface fluxes—are solved simultaneously from the governing equations, allowing interactions and feedback to emerge naturally. Urban versions of RCMs employ detailed parameterizations of building geometry, canyon radiation, impervious surfaces, vegetation, and anthropogenic heat emissions (Kusaka et al., 2001; Martilli et al., 2002; Best et al., 2011; Doan and Kusaka, 2019). This enables simulations of heatwave amplification by the urban heat island (UHI), localized flooding from short-duration rainfall, or ventilation pathways within dense city cores.

Numerous case studies highlight the value of direct dynamical downscaling for urban applications. WRF-UCM has been applied in Tokyo to assess how urban form and coastal breezes shape nocturnal UHI intensities and their health impacts (Kusaka et al., 2012). In Beijing, coupled WRF-urban simulations have been used to evaluate the effectiveness of green roofs and reflective surfaces as mitigation strategies under extreme heat (Zhang et al., 2019). In Europe, convection-permitting RCMs have been used to examine how urban land cover modifies the frequency of heavy summer rainfall events (Daniel et al., 2019). These examples demonstrate the potential of the method to support both scientific understanding and practical policy, such as urban planning, heat adaptation, and flood risk management.

The main limitation is computational cost. Running multi-decadal simulations at kilometre scale requires high-performance computing and weeks to months of runtime, restricting ensemble size and scenario coverage. Multi-decadal simulations at 1–3 km resolution demand high-performance computing and restrict ensemble size. Model results are also sensitive to physical parameterizations, particularly convection, turbulence, and surface fluxes, which introduces uncertainty in extremes (Feser et al., 2011; Rummukainen, 2016). Despite these challenges, direct dynamical downscaling remains the benchmark approach in urban climate studies. it underpins CORDEX regional experiments, provides reference datasets for testing novel methods Within HiClimaX, it anchors the ensemble framework by providing physically robust simulations against which other, more efficient methods can be compared.

### (b) Pseudo-global warming

Pseudo-global warming (PGW) experiments represent a flexible variant of dynamical downscaling. Instead of running long continuous simulations, PGW applies large-scale climate change perturbations derived from global climate models (GCMs) to reanalysis-driven weather events (Sato et al., 2007) . The perturbed boundary conditions are then used to force an urbanized regional climate model (e.g., WRF-UCM), allowing the simulation of extreme events such as typhoons, heatwaves, or convective storms under future climate conditions. This method preserves the synoptic setting of real historical events while embedding the thermodynamic changes expected from global warming.

Compared to conventional direct dynamical downscaling, PGW is more computationally efficient because it does not require multi-decadal integrations. Instead, it targets specific high-impact episodes, making it well suited for attribution studies and for stress-testing cities against plausible future extremes. For example, PGW experiments have been applied to investigate projected rainfall intensification from tropical cyclones over Japan (Yoshida et al., 2017), to explore future changes in heatwave characteristics in Europe (Schär et al., 2004), and to examine future flood risk in Asian megacities through modified typhoon simulations (Kawase et al., 2015) and how local convection extreme changes under global warming (Doan et al., 2022b, 2022c, 2022a).

Relative to statistical downscaling or AI-based methods, PGW retains the physical consistency of the governing equations and includes explicit representations of urban morphology and land–atmosphere coupling. However, unlike continuous RCM runs in CORDEX, PGW does not provide long climatologies or time series for trend analysis. Its scope is therefore complementary: where statistical and AI methods provide efficiency, and conventional RCM ensembles deliver broad coverage, PGW delivers detailed process-based insights into how climate change may alter the intensity and impacts of specific urban extremes. Within HiClimaX, PGW serves as a targeted tool to assess event-level risks and to contextualize the storylines produced by other modules.

### (c) Statistical–dynamical downscaling

### (d) Land-surface-physics-based downscaling

Land-surface-physics-based downscaling (LSP-DS) developed by Xue et al., (2024) focuses on the explicit representation of urban land cover and surface energy exchanges rather than simulating the full atmospheric column (Xue et al., 2024, 2025). These approaches employ land surface models such as the High-Resolution Land Data Assimilation System (HRLDAS) or offline versions of the Noah-MP and SURFEX schemes to resolve surface fluxes, temperature, and soil moisture at grid scales of 1 km or finer. By prescribing atmospheric forcing from reanalysis, GCMs, or RCMs, the models isolate the contribution of surface processes to local climate, particularly the urban heat island (UHI) and surface–atmosphere feedbacks.

Compared with conventional dynamical downscaling, LSP-DS is computationally inexpensive, since it does not integrate the full set of atmospheric equations. This efficiency enables multi-decadal simulations and ensemble experiments at very high spatial resolution. For example, HRLDAS has been applied to reproduce fine-scale patterns of urban heat stress in U.S. cities under different land-use scenarios (Zhang et al., 2013), while SURFEX has been used to quantify the contribution of impervious surfaces to nocturnal heat retention in European cities (Masson et al., 2013). Studies in Asia have demonstrated the method’s value for assessing the impacts of rapid urbanization on local thermal environments, such as in Delhi and Guangzhou, where LSP-DS revealed strong sensitivities of surface temperature to urban expansion (Kusaka & Kimura, 2004; Li et al., 2019).

Relative to statistical and AI-based approaches, LSP-DS retains a clear physical basis, with explicit treatment of energy balance and land-surface heterogeneity. However, because it relies on prescribed meteorology, it cannot capture free-atmosphere feedbacks, circulation changes, or convective dynamics, which are strengths of full RCMs. Within HiClimaX, LSP-DS complements other modules by providing detailed, computationally efficient insights into land-surface and urban morphology effects, which are essential for evaluating adaptation measures such as green infrastructure or reflective surfaces.

### (e) Statistical downscaling

Traditional statistical downscaling remains essential for producing rapid projections across long time series, using empirical relationships between large-scale predictors and local variables.

### (f) AI-based downscaling

AI-based downscaling (AI-DS) uses machine learning (ML) and deep learning (DL) techniques to infer fine-scale climate information from coarse-resolution inputs. Similar to statistical downscaling, AI-DS learns statistical mappings between predictors (e.g., large-scale circulation, temperature, humidity) and predictands (e.g., local rainfall or urban heat) from historical data. Popular approaches include convolutional neural networks (CNNs), generative adversarial networks (GANs), and, more recently, transformer architectures (Vandal et al., 2019; Baño-Medina et al., 2020; Wang et al., 2021).

A key advantage of AI-DS is, once trained, models can produce high-resolution outputs rapidly, enabling large ensembles, multi-scenario experiments, or real-time climate services that are impractical with conventional RCMs. Case studies include CNN-based rainfall downscaling in the U.S. Northeast (Vandal et al., 2018), GAN-based generation of high-resolution precipitation over Europe (Stengel et al., 2020), and hybrid AI-statistical frameworks applied to temperature and precipitation extremes in the Mediterranean (Baño-Medina et al., 2021). In the urban context, AI-DS has been used to create city-scale maps of heat stress and air quality from sparse observations, providing critical information for health and adaptation planning (Wang et al., 2021).

Compared with statistical downscaling, AI methods can capture highly nonlinear relationships, allowing them to generalize better to complex land–atmosphere interactions. However, they remain dependent on the quality, representativeness, and length of training datasets. Unlike dynamical models or PGW experiments, AI-DS does not guarantee physical consistency across variables, and transferability to data-sparse regions (common in the Global South) remains a major uncertainty. For this reason, AI-DS is best viewed as complementary: it adds efficiency and detail to ensemble frameworks, while physical credibility is anchored by dynamical methods. Within HiClimaX, AI-DS provides a rapid and adaptive tool for producing probabilistic city-scale projections and for filling observational gaps.

By integrating six complementary modules, HiClimaX produces projections that balance physical realism with methodological diversity. All outputs are standardized to common spatial and temporal grids, which ensures they can be directly compared or merged into multi-method ensembles. As shown schematically in Fig. 1, the system ingests multiple sources of information—including reanalysis, GCM projections, urban parameter datasets, and observations—and channels them through the downscaling modules. Each module can be executed independently or in combination. The resulting outputs are harmonized, post-processed, and aggregated, providing a framework for systematic intercomparison, ensemble integration, and explicit uncertainty quantification.

### 2.2 Data Interfaces

HiClimaX is designed with flexible data ingestion to accommodate the wide range of inputs required for urban climate downscaling. Urban datasets include land-use and land-cover information, building morphology, and anthropogenic heat fluxes derived from remote sensing or local inventories. Large-scale atmospheric conditions are supplied by reanalysis products such as ERA5, FNL, and JRA, which provide initial and boundary conditions for RCM simulations. Future climate forcing is drawn from CMIP6 GCM outputs, which are bias-corrected and interpolated to six-hourly or monthly time steps. For evaluation, observational products such as CMORPH and IMERG, together with in situ station data, are incorporated. A common preprocessing pipeline harmonizes these inputs into formats compatible with all downscaling modules, ensuring methodological comparability.

### 2.3 Workflow and Implementation

The workflow of HiClimaX is organized into four stages. First, raw GCM, reanalysis, and urban datasets are preprocessed and standardized. Second, one or more downscaling modules are executed to generate high-resolution fields. Third, post-processing steps such as bias correction, regridding, and ensemble integration are applied. Finally, outputs are analysed and visualized through maps, time series, and event diagnostics. The framework is implemented using a combination of Fortran (for model execution) and Python (for workflow automation, data handling, and visualization). All scripts and configuration files are maintained in a public repository to ensure transparency and reproducibility.

# 3. Case Studies and Demonstration

To demonstrate the capabilities of HiClimaX, we configured a set of case studies spanning three urban regions: Tokyo (Japan), Ho Chi Minh City (Vietnam), and Brussels (Belgium). These cities were chosen to represent distinct climatic regimes and urbanization patterns: a temperate megacity strongly influenced by mid-latitude synoptic systems (Tokyo), a tropical monsoon metropolis subject to convective rainfall (Ho Chi Minh City), and a mid-latitude European city where moderate-scale urbanization interacts with Atlantic and continental air masses (Brussels). Together, these case studies provide a diverse testbed for evaluating the transferability of HiClimaX across contexts.

### 3.1 Experimental Design

Each case study was configured with a nested regional domain centred on the city of interest. The horizontal resolution for dynamical and physics-based modules was set to 1–3 km, sufficient to resolve major urban districts and capture mesoscale processes such as sea breezes and convective initiation. For statistical and AI-based methods, outputs were produced on grids of comparable resolution, allowing direct intercomparison with dynamical simulations. The baseline evaluation period was defined as 1991–2020, consistent with the availability of reanalysis and observational datasets. In addition, targeted event simulations were carried out for selected extreme episodes, including heatwaves, intense rainfall events, and typhoon passages, to assess module performance under high-impact conditions.

### 3.2 Input Datasets

Large-scale forcing for dynamical downscaling was provided by ERA5 and JRA reanalyses for historical simulations, and by CMIP6 global climate model outputs for future scenario experiments. The latter were bias-corrected at six-hourly or monthly intervals prior to use. Urban parameter datasets, including land-use/land-cover, anthropogenic heat fluxes, and building morphology, were derived from remote sensing products and national inventories specific to each city. Observational datasets (IMERG, CMORPH, and local station records) were used for evaluation and bias correction. All datasets were preprocessed through the HiClimaX interface, ensuring consistency in spatial resolution, temporal alignment, and variable naming conventions.

### 3.3 Demonstration Strategy

The case studies were not intended to deliver exhaustive climate change projections, but rather to illustrate the modularity, comparability, and ensemble capabilities of HiClimaX. For each city, outputs from multiple downscaling modules were generated and intercompared. Illustrative diagnostics included spatial maps of seasonal mean temperature and precipitation, time series of urban heat island intensity, and event-based composites of extreme rainfall. Ensemble products were used to quantify inter-method variability and to demonstrate how uncertainty can be communicated using probability distributions and storyline representations.

### 3.4 Proof-of-Concept Evaluation

Preliminary results confirm the ability of HiClimaX to reproduce key urban climate features. For example, the dynamical modules captured the spatial heterogeneity of temperature associated with land–sea contrasts in Tokyo and Ho Chi Minh City, while AI-based modules reproduced fine-scale rainfall structures at reduced computational cost. Statistical modules provided efficient multi-decadal time series suitable for uncertainty assessment, although with less spatial detail. The ensemble framework successfully highlighted divergences among methods, such as differences in the magnitude of extreme rainfall intensities, underscoring the importance of a multi-method approach.

Overall, the case studies demonstrate that HiClimaX can be applied across contrasting climates and urban contexts, producing standardized, comparable outputs that highlight both the strengths and limitations of individual downscaling families. These proof-of-concept experiments establish the foundation for broader multi-city applications and systematic intercomparison studies.

# 4. Evaluation and Results

The primary objective of this section is not to deliver exhaustive urban climate projections, but rather to demonstrate the ability of HiClimaX to produce consistent, comparable outputs across multiple downscaling approaches. The evaluation focuses on two aspects: (i) the reproduction of salient urban climate features under present-day conditions, and (ii) the added value of multi-method ensembles for quantifying and communicating uncertainty.

### 4.1 Reproduction of Urban Climate Features

Across all three case studies, HiClimaX successfully reproduced major characteristics of the urban climate. Dynamical simulations with WRF-UCM and RegCM-urban captured the spatial variability of near-surface temperature associated with urban morphology, coastlines, and topography. For example, summertime runs over Tokyo reproduced nocturnal urban heat island (UHI) intensities of 2–3 °C, consistent with local station observations. AI-based downscaling produced comparable spatial structures in temperature and precipitation fields, with markedly reduced computational costs. In Ho Chi Minh City, the AI modules replicated convective rainfall maxima associated with the monsoon trough, although extreme rainfall intensities were somewhat underestimated relative to dynamical simulations. Statistical approaches reproduced seasonal cycles and long-term mean fields, offering efficient multi-decadal coverage, but lacked spatial detail at the neighbourhood scale.

### 4.2 Cross-Method Comparison

The comparison of modules highlighted both complementarities and divergences. For temperature, inter-method agreement was generally high in terms of mean seasonal values, with differences of less than 0.5 °C among methods. For precipitation, however, larger discrepancies emerged, particularly for heavy rainfall events. Dynamical modules tended to simulate more intense short-duration extremes, while statistical and AI methods smoothed these peaks. Such differences underscore the need for multi-method ensembles, as reliance on a single technique could bias impact assessments.

### 4.3 Ensemble Integration and Uncertainty

Ensemble products aggregated across the six modules provided a transparent means of quantifying uncertainty. In all cities, ensemble spreads for daily mean temperature were relatively narrow (±0.5 °C), whereas spreads for daily precipitation were wider (±20–40 %). Probability density functions of precipitation extremes revealed method-dependent tails, with dynamical simulations producing higher return-level estimates than statistical approaches. Storyline representations further illustrated plausible but divergent outcomes for event-based simulations: for example, a pseudo-global warming experiment in Tokyo suggested a 15–20 % increase in extreme rainfall under future warming, while AI-based downscaling indicated more modest increases of 5–10 %.

### 4.4 Proof-of-Concept Insights

These results demonstrate the capacity of HiClimaX to generate high-resolution, urban-relevant climate information while making uncertainties explicit. The framework shows that no single method is universally superior: dynamical models are indispensable for process studies, statistical and AI approaches provide efficiency and scalability, and physics-based modules highlight land–surface feedbacks. By integrating all approaches, HiClimaX offers a balanced view that acknowledges methodological diversity and provides more robust information for downstream applications.

# 5. Discussion

The development of HiClimaX addresses several persistent challenges in urban climate downscaling. By integrating six methodological families within a single modular framework, the system moves beyond the fragmented landscape of existing approaches, where studies typically rely on one or two methods applied in isolation. The proof-of-concept experiments demonstrate that HiClimaX can reproduce salient urban climate features across diverse environments and, more importantly, provide a structured basis for inter-method comparison and uncertainty quantification.

A key advantage of HiClimaX is that it allows a direct comparison of downscaling methods within a unified system. Table 1 summarizes the main strengths and weaknesses of the six approaches currently implemented. Dynamical simulations (D-DS, PGW-DS) are indispensable for resolving processes such as convection, urban heat islands, and circulation feedbacks, but their computational demands limit the length and number of simulations. Statistical methods, by contrast, are computationally inexpensive and scalable, but rely on stationarity assumptions that may fail under climate change. Physics-based approaches (LSP-DS) capture land–atmosphere feedbacks, while AI-based downscaling provides efficiency and adaptability but raises questions of generalizability and interpretability. The combination of these approaches in a single ensemble framework ensures that the limitations of one method are balanced by the strengths of others.

Table 1. Advantages and disadvantages of the six downscaling approaches implemented in HiClimaX.

| Method | Advantages | Disadvantages |
| --- | --- | --- |
| Direct Dynamical Downscaling (D-DS) | Physically consistent; resolves mesoscale processes (e.g. convection, sea breezes); well-established community models (WRF, RegCM). | Very high computational cost; sensitive to parameterizations; limited feasibility for long multi-decadal ensembles. |
| Pseudo-Global Warming (PGW-DS) | Isolates climate-change signal in extreme events; flexible for event attribution; straightforward experimental design. | Does not provide continuous time series; depends strongly on GCM perturbation patterns; not suitable for long climatologies. |
| Statistical–Dynamical Downscaling (SD-DS) | Computationally efficient compared to pure dynamical methods; retains circulation–climate relationships; suitable for large ensembles. | Requires classification schemes; may underperform for unprecedented circulation states; less process fidelity than full dynamical models. |
| Land-Surface-Physics-Based Downscaling (LSP-DS) | Explicit representation of surface fluxes, morphology, and anthropogenic heat; highlights land–atmosphere feedbacks; valuable for UHI and heatwave studies. | Limited to near-surface processes; requires detailed urban data; lacks free atmosphere dynamics, so cannot capture circulation feedbacks. |
| Statistical Downscaling (SDS) | Very fast and computationally inexpensive; provides long time series; simple to implement; useful for multi-model ensembles. | Assumes stationarity of predictor–predictand relationships; limited ability to represent extremes; less spatial detail. |
| AI-Based Downscaling (AI-DS) | High computational efficiency once trained; captures complex nonlinear patterns; adaptable to multiple cities; scalable. | Requires large training datasets; generalizability uncertain outside training domain; risk of black-box behaviour; dependent on training quality. |

### Table X. Cross-comparison of downscaling methods in HiClimaX by key attributes.

| Attribute/ Method | Direct Dynamical (D-DS) | PGW Experiments (PGW-DS) | Statistical–Dynamical (SD-DS) | Land-Surface Physics (LSP-DS) | Statistical (SDS) | AI-Based (AI-DS) |
| --- | --- | --- | --- | --- | --- | --- |
| Physical consistency | ★★★★★ | ★★★★☆ | ★★★☆☆ | ★★★★☆ | ★★☆☆☆ | ★★★☆☆ |
| Computational cost | Very high | High (event-based) | Moderate | Low | Very low | Very low (after training) |
| Data requirements | Reanalysis + GCM + urban parameters | Same as D-DS + perturbation fields | Reanalysis + GCM | High-resolution urban data (LUC, AH, morphology) | Predictor–predictand pairs | Large training datasets |
| Ability to capture extremes | Excellent | Excellent (for events) | Good | Moderate (surface-driven) | Limited | Good (pattern recognition) |
| Scalability (multi-city, long runs) | Low | Low–moderate | High | Moderate | Very high | High |
| Transparency & interpretability | High | High | Moderate | High | High | Low–moderate |
| Innovation potential | Mature, well-established | Useful for attribution | Hybrid flexibility | Novel for UHI/flux studies | Widely applied | Rapidly evolving, high potential |

Several limitations must also be acknowledged. First, the computational requirements of dynamical modules remain substantial, restricting the feasibility of large ensemble or multi-decadal simulations at kilometre-scale resolution. While AI and statistical modules mitigate this cost, they cannot yet fully replicate the process fidelity of dynamical simulations. Second, urban data availability and quality remain a bottleneck. Many cities lack reliable inventories of land use, morphology, and anthropogenic heat emissions. Uncertainties in these inputs propagate into all downscaling modules, reducing confidence in results, especially for poorly monitored regions. Third, the system’s evaluation remains limited to proof-of-concept demonstrations. More comprehensive validation against multi-variable observational datasets, including surface flux measurements and urban climate networks, is necessary to establish the general performance of HiClimaX across different climates and scales.

HiClimaX opens several promising avenues for future work. Its ensemble-based design aligns with the objectives of the CORDEX initiative and ongoing efforts within the WCRP community to improve intercomparison of downscaling techniques. By providing a structured testbed, HiClimaX can contribute to benchmarking exercises and the development of best practices for urban climate downscaling. The integration of AI-based methods within a physically informed framework creates opportunities for hybrid modelling, such as machine learning emulators trained on dynamical simulations to accelerate ensemble generation. In addition, the modular structure of HiClimaX facilitates coupling with urban digital twins and decision-support platforms, where high-resolution climate inputs must be combined with socio-economic and infrastructural data. Finally, the open and transparent design of the system, including access to source code and processing routines, contributes to closing the reproducibility gap in complex climate modelling studies.

# 6. Conclusions and Outlook

This paper has introduced HiClimaX, a modular and multi-method framework for high-resolution climate downscaling. By integrating six distinct methodological families—dynamical, pseudo-global warming, statistical–dynamical, land-surface-physics-based, statistical, and AI-based approaches—HiClimaX provides the first comprehensive platform to systematically compare, combine, and evaluate downscaling methods within a single ensemble structure. The framework emphasizes transparency, reproducibility, and extensibility, thereby addressing long-standing gaps in the fragmented practice of urban climate downscaling.

Proof-of-concept demonstrations in Tokyo, Ho Chi Minh City, and Brussels confirmed that HiClimaX can reproduce salient urban climate features and highlight inter-method divergences. These case studies illustrate both the complementarities and trade-offs among methods, underscoring the value of an ensemble-based approach for quantifying and communicating uncertainty.

Looking forward, several priorities emerge. First, comprehensive validation against multi-variable observational networks is required to establish the robustness of HiClimaX across different climates and urban forms. Second, further development of hybrid approaches, particularly the integration of AI methods with process-based models, offers opportunities to balance computational efficiency with physical realism. Third, expansion of the framework to additional cities and regions will allow broader intercomparison exercises, aligning HiClimaX with the objectives of CORDEX and other WCRP initiatives. Finally, the open-source design of HiClimaX provides a foundation for coupling with digital twin platforms and urban climate services, supporting more actionable climate information for adaptation and resilience planning.

In summary, HiClimaX represents a step change in urban climate modelling methodology: from isolated and method-specific applications toward a unified, transparent, and ensemble-based framework. Its continued development and application will strengthen the credibility, comparability, and accessibility of high-resolution urban climate projections, advancing both scientific understanding and their use in practice.

# Code and Data Availability

- links (Zenodo/GitHub).

- Details on datasets used.

# Author Contributions

# Competing Interests

# Acknowledgements

(funding, collaborations, institutions)

Figure 1: An example for accurate data representation & universal readability of figures.

# References

Devadarshini E, Geethalakshmi V, McDermid SP, Bhuvaneswari K, Mohan Kumar S, Sathyamoorthy NK, Senthilnathan S, Senthilraja K, Anandhi V. 2025. A systematic review of climate downscaling and extremes in Coupled Model Intercomparison Project 6 (CMIP6). Environmental Development, 56: 101280. https://doi.org/10.1016/j.envdev.2025.101280.

Xue L, Doan Q-V, Kusaka H, He C, Chen F. 2025. Land-surface-physics-based downscaling versus conventional dynamical downscaling for high-resolution urban climate change information: The case study of two cities. Urban Climate, 59: 102228. https://doi.org/10.1016/j.uclim.2024.102228.

Xue L, Doan Q-V, Kusaka H, He C, Chen F. 2024. Insights Into Urban Heat Island and Heat Waves Synergies Revealed by a Land-Surface-Physics-Based Downscaling Method. Journal of Geophysical Research: Atmospheres, 129(13): e2023JD040531. https://doi.org/10.1029/2023JD040531.

Lafferty DC, Sriver RL. 2023. Downscaling and bias-correction contribute considerable uncertainty to local climate projections in CMIP6. npj Climate and Atmospheric Science. Nature Publishing Group, 6(1): 158. https://doi.org/10.1038/s41612-023-00486-0.

Mishra AK, Jangir B, Strobach E. 2023. Does Increasing Climate Model Horizontal Resolution Be Beneficial for the Mediterranean Region?: Multimodel Evaluation Framework for High-Resolution Model Intercomparison Project. Journal of Geophysical Research: Atmospheres, 128(2): e2022JD037812. https://doi.org/10.1029/2022JD037812.

Doan Q-V, Chen F, Asano Y, Gu Y, Nishi A, Kusaka H, Niyogi D. 2022a. Causes for Asymmetric Warming of Sub-Diurnal Temperature Responding to Global Warming. Geophysical Research Letters, 49(20): e2022GL100029. https://doi.org/10.1029/2022GL100029.

Doan Q-V, Chen F, Kusaka H, Dipankar A, Khan A, Hamdi R, Roth M, Niyogi D. 2022b. Increased Risk of Extreme Precipitation Over an Urban Agglomeration With Future Global Warming. Earth’s Future, 10(6): e2021EF002563. https://doi.org/10.1029/2021EF002563.

Doan Q-V, Chen F, Kusaka H, Wang J, Kajino M, Takemi T. 2022c. Identifying a New Normal in Extreme Precipitation at a City Scale Under Warmer Climate Regimes: A Case Study of the Tokyo Metropolitan Area, Japan. Journal of Geophysical Research: Atmospheres, 127(21): e2022JD036810. https://doi.org/10.1029/2022JD036810.

IPCC AR6. 2022. AR6 Synthesis Report: Climate Change 2022 — IPCC. .

Doan Q-V, Chen F, Kusaka H, Khan A, Dipankar A, Hamdi R, Gupta A, Niyogi D. 2021. Response of Extreme Convective Rainfall in a Tropical City to the Future Warming Climate. paper presented at the AOGS 2021 18th Annual Meeting.

Tabari H, Paz SM, Buekenhout D, Willems P. 2021. Comparison of statistical downscaling methods for climate change impact analysis on precipitation-driven drought. Hydrology and Earth System Sciences. Copernicus GmbH, 25(6): 3493–3517. https://doi.org/10.5194/hess-25-3493-2021.

Iles CE, Vautard R, Strachan J, Joussaume S, Eggen BR, Hewitt CD. 2020. The benefits of increasing resolution in global and regional climate simulations for European climate extremes. Geoscientific Model Development. Copernicus GmbH, 13(11): 5583–5607. https://doi.org/10.5194/gmd-13-5583-2020.

Masson V, Heldens W, Bocher E, Bonhomme M, Bucher B, Burmeister C, de Munck C, Esch T, Hidalgo J, Kanani-Sühring F, Kwok Y-T, Lemonsu A, Lévy J-P, Maronga B, Pavlik D, Petit G, See L, Schoetter R, Tornay N, Votsis A, Zeidler J. 2020. City-descriptive input data for urban climate models: Model requirements, data sources and challenges. Urban Climate, 31: 100536. https://doi.org/10.1016/j.uclim.2019.100536.

Zhang L, Xu Y, Meng C, Li X, Liu H, Wang C. 2020. Comparison of Statistical and Dynamic Downscaling Techniques in Generating High-Resolution Temperatures in China from CMIP5 GCMs. . https://doi.org/10.1175/JAMC-D-19-0048.1.

Doan VQ, Kusaka H. 2019. Development of a Multilayer Urban Canopy Model Combined with a Ray Tracing Algorithm. Scientific Online Letters on the Atmosphere, 15: 37–40. https://doi.org/10.2151/sola.2019-008.

Giorgi F. 2019. Thirty Years of Regional Climate Modeling: Where Are We and Where Are We Going next? Journal of Geophysical Research: Atmospheres, 124(11): 5696–5723. https://doi.org/10.1029/2018JD030094.

Shepherd TG, Boyd E, Calel RA, Chapman SC, Dessai S, Dima-West IM, Fowler HJ, James R, Maraun D, Martius O, Senior CA, Sobel AH, Stainforth DA, Tett SFB, Trenberth KE, van den Hurk BJJM, Watkins NW, Wilby RL, Zenghelis DA. 2018. Storylines: an alternative approach to representing uncertainty in physical aspects of climate change. Climatic Change, 151(3): 555–571. https://doi.org/10.1007/s10584-018-2317-9.

Oke TR, Mills G, Christen A, Voogt JA. 2017. Urban climates. Cambridge University Press.

Prein AF, Rasmussen R, Stephens G. 2017. Challenges and Advances in Convection-Permitting Climate Modeling. Bulletin of the American Meteorological Society. American Meteorological Society, 98(5): 1027–1030. https://doi.org/10.1175/BAMS-D-16-0263.1.

Rummukainen M. 2016. Added value in regional climate modeling. WIREs Climate Change, 7(1): 145–159. https://doi.org/10.1002/wcc.378.

Sailor DJ, Georgescu M, Milne JM, Hart MA. 2015. Development of a national anthropogenic heating database with an extrapolation for international cities. Atmospheric Environment. Elsevier Ltd, 118(2015): 7–18. https://doi.org/10.1016/j.atmosenv.2015.07.016.

Hamdi R, Van de Vyver H, De Troch R, Termonia P. 2014. Assessment of three dynamical urban climate downscaling methods: Brussels’s future urban heat island under an A1B emission scenario. International Journal of Climatology, 34(4): 978–999. https://doi.org/10.1002/joc.3734.

Gutiérrez JM, San-Martín D, Brands S, Manzanas R, Herrera S. 2013. Reassessing Statistical Downscaling Techniques for Their Robust Application under Climate Change Conditions. Journal of Climate. American Meteorological Society, 26(1): 171–188. https://doi.org/10.1175/JCLI-D-11-00687.1.

Wilby RL, Dawson CW. 2013. The Statistical DownScaling Model: insights from one decade of application. International Journal of Climatology, 33(7): 1707–1719. https://doi.org/10.1002/joc.3544.

Best MJ, Pryor M, Clark DB, Rooney GG, Essery RLH, Ménard CB, Edwards JM, Hendry MA, Porson A, Gedney N, Mercado LM, Sitch S, Blyth E, Boucher O, Cox PM, Grimmond CSB, Harding RJ. 2011. The Joint UK Land Environment Simulator (JULES), model description – Part 1: Energy and water fluxes. Geoscientific Model Development. Copernicus GmbH, 4(3): 677–699. https://doi.org/10.5194/gmd-4-677-2011.

Grimmond CSB, Roth M, Oke TR, Au YC, Best M, Betts R, Carmichael G, Cleugh H, Dabberdt W, Emmanuel R, Freitas E, Fortuniak K, Hanna S, Klein P, Kalkstein LS, Liu CH, Nickson A, Pearlmutter D, Sailor D, Voogt J. 2010. Climate and More Sustainable Cities: Climate Information for Improved Planning and Management of Cities (Producers/Capabilities Perspective). Procedia Environmental Sciences, 1: 247–274. https://doi.org/10.1016/j.proenv.2010.09.016.

Stone B, Hess JJ, Frumkin H. 2010. Urban Form and Extreme Heat Events: Are Sprawling Cities More Vulnerable to Climate Change Than Compact Cities? Environmental Health Perspectives. Environmental Health Perspectives, 118(10): 1425–1428. https://doi.org/10.1289/ehp.0901879.

Skamarock WC, Klemp JB, Dudhia J, Gill DO, Barker DM, Wang W, Powers JG. 2008. A description of the Advanced Research WRF version 3. NCAR Technical note -475+STR. .

Sato T, Kimura F, Kitoh A. 2007. Projection of global warming onto regional precipitation over Mongolia using a regional climate model. Journal of Hydrology, 333(1): 144–154. https://doi.org/10.1016/j.jhydrol.2006.07.023.

Shepherd JM, Burian SJ. 2003. Detection of Urban-Induced Rainfall Anomalies in a Major Coastal City. .

Martilli A, Clappier A, Rotach MW. 2002. An Urban Surface Exchange Parameterisation for Mesoscale Models. Boundary-Layer Meteorology, 104(2): 261–304. https://doi.org/10.1023/A:1016099921195.

Kusaka H, Kondo H, Kikegawa Y, Kimura F. 2001. A Simple Single-Layer Urban Canopy Model For Atmospheric Models: Comparison With Multi-Layer And Slab Models. Boundary-Layer Meteorology, 101(3): 329–358. https://doi.org/10.1023/A:1019207923078.